# Concurrency & Latency Analysis Report

Category A LLM Models on NVIDIA H100 SXM 80GB with vLLM
Analytical predictions — empirical results in Appendix C.

# 1. Executive Summary

This report analyses the maximum concurrent-request capacity for each of the 12 Category A models (15–35B parameters) served on a single NVIDIA H100 SXM 80GB GPU via vLLM. Concurrency is bounded primarily by KV cache VRAM after model weights are loaded. Key levers are:

KV cache dtype: BF16 (1×) → FP8 (2×) → TurboQuant 3-bit (~5.3×) compression.

Weight dtype: FP8 checkpoint variants free 20–38 GB of additional VRAM.

Architecture: MoE active params (3–4B) leave more headroom than dense models; MLA (GLM-4.7-Flash) compresses KV cache ~4× vs GQA baseline.

Model-level vLLM constraints: Gemma4 (all variants) and Nemotron-3-Nano are TurboQuant-incompatible (see Section 5.2 and CLAUDE.md R6/R8).

Predicted maximum concurrent sessions at ctx=4096 (conservative lower bound):

| Model | Weight VRAM | BF16 KV (ctx 4096) | FP8 KV (ctx 4096) | TQ 3-bit | Notes |
| --- | --- | --- | --- | --- | --- |
| Gemma 3 27B-IT | 54 GB | 4 | 9 | 24 |  |
| Gemma 4 26B-A4B-IT (MoE, BF16) | 52 GB | 13 | 27 | N/A |  |
| Gemma 4 26B-A4B-IT (MoE, FP8 weights) | 26 GB | 32 | 64 | N/A |  |
| Gemma 4 31B-IT (Dense, BF16) | 62 GB | 5 | 10 | N/A |  |
| Gemma 4 31B-IT (Dense, FP8 block weights) | 32 GB | 20 | 41 | N/A |  |
| GLM-4.7-Flash (30B / 3.6B active MoE, MLA) | 60 GB | 25 | 50 | 134 |  |
| Mistral Small 3.1 24B-Instruct | 48 GB | 38 | 76 | 204 |  |
| Nemotron-3-Nano 30B-Instruct (Mamba hybrid) | 60 GB | 21 | 42 | N/A |  |
| Qwen3.5-35B-A3B (DeltaNet + MoE, BF16) | 70 GB | 16 | 32 | 85 |  |
| Qwen3.6-35B-A3B (DeltaNet + MoE, BF16) | 70 GB | 16 | 32 | 85 |  |
| Qwen3.6-35B-A3B (DeltaNet + MoE, FP8 weights) | 35 GB | 296 | 592 | 1578 |  |
| Qwen3-32B | 64 GB | 8 | 16 | 42 |  |

# 2. H100 SXM 80GB Hardware Envelope

The NVIDIA H100 SXM 80GB is the target hardware for all experiments. Key specifications that constrain LLM serving throughput and concurrency:

| Parameter | Value |
| --- | --- |
| Total VRAM | 80 GB HBM3 |
| Memory bandwidth | 3.35 TB/s |
| BF16 Tensor Core throughput | 989 TFLOPS |
| FP8 Tensor Core throughput | 1,979 TFLOPS (2× BF16) |
| NVLink bandwidth | 900 GB/s bidirectional (multi-GPU, N/A single-GPU) |
| SM count | 132 Streaming Multiprocessors |
| Usable VRAM at gpu_memory_utilization=0.90 | 72 GB |

Decode-phase bottleneck: For models >15B parameters, the bottleneck during the auto-regressive decode phase is memory bandwidth, not compute.  Each decode step reads the entire KV cache plus model weights from HBM — arithmetic intensity falls below the BF16 compute/bandwidth ridge point (~74 FLOPS/byte) for batch sizes below ~500 concurrent tokens.  Increasing concurrency raises arithmetic intensity and improves hardware utilisation until the memory capacity (not bandwidth) becomes the ceiling.

Prefill-phase: For long prompts (>1K tokens) the workload is compute-bound on H100.  TTFT is dominated by prompt processing latency, which scales linearly with prompt length and inversely with batch throughput.

# 3. vLLM Scheduling and KV Cache Quantisation

## 3.1  PagedAttention and Continuous Batching

vLLM allocates KV cache in fixed-size pages (default 16 tokens/block).  Sequences share the physical KV block pool — no over-allocation, no fragmentation.  Continuous batching inserts new requests into decode batches as soon as a slot frees, maximising GPU utilisation.  Maximum in-flight sequences is limited by the KV block pool size, which depends on:

gpu_memory_utilization × VRAM_total − model_weight_memory = KV block pool

block_size (tokens per block) × bytes_per_block = KV block size

max_model_len controls the maximum KV cache a single sequence may consume

All Cat A configs set gpu_memory_utilization=0.90 (72 GB usable) and max_model_len=8192.  The benchmark script overrides max_model_len at launch to accommodate the largest requested context length plus output tokens.

## 3.2  KV Cache Dtype Options and Memory Multipliers

| Dtype | Memory multiplier | Storage | Notes |
| --- | --- | --- | --- |
| auto / BF16 | 1.0× | 2 bytes per element | Full precision KV; most headroom consumed |
| FP8 (--kv-cache-dtype fp8) | 0.50× | 1 byte per element | Native vLLM support; ~0–1% quality drop; 2× sessions |
| KIVI 2-bit | 0.125× | 0.25 bytes per element | Asymmetric per-channel; no calibration needed |
| KIVI 4-bit | 0.25× | 0.5 bytes per element | Higher quality than 2-bit; same no-calibration property |
| KVQuant NUQ | 0.125–0.25× | variable | NUQ codebooks; calibration required; dense-sparse decomp |
| TurboQuant 3-bit | ≈0.19× | 3 bits per elem (packed) | Codebook + rotation + QJL residual; ~5.3× vs BF16 |
| TurboQuant 4-bit | ≈0.25× | 4 bits per elem | Higher quality than 3-bit variant |

TurboQuant incompatibility: Gemma4 (all variants) forces TRITON_ATTN in vLLM due to heterogeneous head dimensions (head_dim mixes 256 and 128 across global/local layers).  TRITON_ATTN does not accept turboquant_* dtypes.  Nemotron-3-Nano's Mamba hybrid raises NotImplementedError in TurboQuant's boundary-layer protection.  Both model families are limited to FP8 / KIVI / KVQuant for KV compression.

## 3.3  Benchmark Degradation Policy

The empirical benchmark (scripts/run_concurrency_benchmark.sh) defines 'no degradation' as the highest concurrency level N satisfying both:

(1)  TTFT p95[N]  ≤  2.0 × TTFT p95[concurrency=1]

(2)  Request failure rate[N]  ≤  1 %

The full concurrency curve is recorded (not a binary search), so latency vs. concurrency plots can be generated.  Context lengths swept: 2048, 4096, 8192 tokens.

# 4. Per-Model Analysis

Each sub-section presents the architecture card, KV cache memory formula, available VRAM budget, and predicted maximum concurrent sessions at ctx=4096 for BF16 and FP8 KV dtypes.  Numbers are analytical lower bounds; vLLM's paged allocator allows higher effective concurrency when average session context is shorter than the maximum.

## 4.x  Gemma 3 27B-IT

| Parameter | Value |
| --- | --- |
| Full model name | google/gemma-3-27b-it |
| Architecture type | Dense GQA Transformer |
| Attention detail | GQA, alternating local (sliding-window 1024) + global layers |
| KV-contributing layers | 62 |
| KV heads per layer | 16 |
| Head dimension | 256 |
| MoE | No |
| Mamba hybrid | No |
| MLA | No |
| TurboQuant compatible | Yes |
| Weight VRAM (BF16) | 54 GB |
| Confidence | HIGH |
| Source | Gemma 3 Technical Report; HF config google/gemma-3-27b-it |

KV cache budget at gpu_memory_utilization=0.90:  72 GB − 54 GB = 18.0 GB

KV dtype BF16: 992.0 KB/token × 2048 tokens = 1938 MB/session → max 9 concurrent sessions

KV dtype BF16: 992.0 KB/token × 4096 tokens = 3875 MB/session → max 4 concurrent sessions

KV dtype BF16: 992.0 KB/token × 8192 tokens = 7750 MB/session → max 2 concurrent sessions

KV dtype FP8: 496.0 KB/token × 2048 tokens = 969 MB/session → max 18 concurrent sessions

KV dtype FP8: 496.0 KB/token × 4096 tokens = 1938 MB/session → max 9 concurrent sessions

KV dtype FP8: 496.0 KB/token × 8192 tokens = 3875 MB/session → max 4 concurrent sessions

Note: BF16 weights only; no official FP8 checkpoint

## 4.x  Gemma 4 26B-A4B-IT (MoE, BF16)

| Parameter | Value |
| --- | --- |
| Full model name | google/gemma-4-26B-A4B-it |
| Architecture type | MoE GQA Transformer (256 experts, top-4 routing) |
| Attention detail | GQA with heterogeneous head_dim: global layers head_dim=256, local layers head_dim=128 |
| KV-contributing layers | 46 |
| KV heads per layer | 8 |
| Head dimension | 256 |
| MoE | Yes |
| Mamba hybrid | No |
| MLA | No |
| TurboQuant compatible | No |
| Weight VRAM (BF16) | 52 GB |
| Confidence | ESTIMATED |
| Source | Gemma 4 architecture inferred from model card and vLLM source comments |

KV cache budget at gpu_memory_utilization=0.90:  72 GB − 52 GB = 20.0 GB

KV dtype BF16: 368.0 KB/token × 2048 tokens = 719 MB/session → max 27 concurrent sessions

KV dtype BF16: 368.0 KB/token × 4096 tokens = 1438 MB/session → max 13 concurrent sessions

KV dtype BF16: 368.0 KB/token × 8192 tokens = 2875 MB/session → max 6 concurrent sessions

KV dtype FP8: 184.0 KB/token × 2048 tokens = 359 MB/session → max 55 concurrent sessions

KV dtype FP8: 184.0 KB/token × 4096 tokens = 719 MB/session → max 27 concurrent sessions

KV dtype FP8: 184.0 KB/token × 8192 tokens = 1438 MB/session → max 13 concurrent sessions

Note: vLLM forces TRITON_ATTN (heterogeneous head_dim > 256 mixed with 128 local layers). TRITON_ATTN does not support turboquant_* — see CLAUDE.md R8.

Note: BF16 weights; use FP8 variant for lower VRAM

## 4.x  Gemma 4 26B-A4B-IT (MoE, FP8 weights)

| Parameter | Value |
| --- | --- |
| Full model name | RedHatAI/gemma-4-26B-A4B-it-FP8-Dynamic |
| Architecture type | MoE GQA Transformer (256 experts, top-4 routing) — FP8 weights |
| Attention detail | Same as BF16 variant; FP8 is weight quantization (activations BF16) |
| KV-contributing layers | 46 |
| KV heads per layer | 8 |
| Head dimension | 256 |
| MoE | Yes |
| Mamba hybrid | No |
| MLA | No |
| TurboQuant compatible | No |
| Weight VRAM (BF16) | 26 GB |
| Confidence | ESTIMATED |
| Source | RedHatAI FP8-Dynamic checkpoint; architecture unchanged from BF16 base |

KV cache budget at gpu_memory_utilization=0.90:  72 GB − 26 GB = 46.0 GB

KV dtype BF16: 368.0 KB/token × 2048 tokens = 719 MB/session → max 64 concurrent sessions

KV dtype BF16: 368.0 KB/token × 4096 tokens = 1438 MB/session → max 32 concurrent sessions

KV dtype BF16: 368.0 KB/token × 8192 tokens = 2875 MB/session → max 16 concurrent sessions

KV dtype FP8: 184.0 KB/token × 2048 tokens = 359 MB/session → max 128 concurrent sessions

KV dtype FP8: 184.0 KB/token × 4096 tokens = 719 MB/session → max 64 concurrent sessions

KV dtype FP8: 184.0 KB/token × 8192 tokens = 1438 MB/session → max 32 concurrent sessions

Note: Same TRITON_ATTN constraint as BF16 variant (R8); FP8 KV cache also blocked.

Note: FP8 dynamic weight quantization halves weight VRAM (~26 GB); KV cache still BF16 or FP8

## 4.x  Gemma 4 31B-IT (Dense, BF16)

| Parameter | Value |
| --- | --- |
| Full model name | google/gemma-4-31B-it |
| Architecture type | Dense GQA Transformer |
| Attention detail | GQA with heterogeneous head_dim (same family as 26B-A4B) |
| KV-contributing layers | 62 |
| KV heads per layer | 8 |
| Head dimension | 256 |
| MoE | No |
| Mamba hybrid | No |
| MLA | No |
| TurboQuant compatible | No |
| Weight VRAM (BF16) | 62 GB |
| Confidence | ESTIMATED |
| Source | Inferred from YAML vram_estimate_gb=62 and Gemma4 architecture family |

KV cache budget at gpu_memory_utilization=0.90:  72 GB − 62 GB = 10.0 GB

KV dtype BF16: 496.0 KB/token × 2048 tokens = 969 MB/session → max 10 concurrent sessions

KV dtype BF16: 496.0 KB/token × 4096 tokens = 1938 MB/session → max 5 concurrent sessions

KV dtype BF16: 496.0 KB/token × 8192 tokens = 3875 MB/session → max 2 concurrent sessions

KV dtype FP8: 248.0 KB/token × 2048 tokens = 484 MB/session → max 20 concurrent sessions

KV dtype FP8: 248.0 KB/token × 4096 tokens = 969 MB/session → max 10 concurrent sessions

KV dtype FP8: 248.0 KB/token × 8192 tokens = 1938 MB/session → max 5 concurrent sessions

Note: vLLM forces TRITON_ATTN (heterogeneous head_dim) — same constraint as A4B variants (R8).

Note: BF16 weights; ~62 GB VRAM for weights alone — very tight on H100 80GB

## 4.x  Gemma 4 31B-IT (Dense, FP8 block weights)

| Parameter | Value |
| --- | --- |
| Full model name | RedHatAI/gemma-4-31B-it-FP8-block |
| Architecture type | Dense GQA Transformer — FP8 block-quantized weights |
| Attention detail | Same architecture as BF16 variant; block FP8 for weights |
| KV-contributing layers | 62 |
| KV heads per layer | 8 |
| Head dimension | 256 |
| MoE | No |
| Mamba hybrid | No |
| MLA | No |
| TurboQuant compatible | No |
| Weight VRAM (BF16) | 32 GB |
| Confidence | ESTIMATED |
| Source | RedHatAI FP8-block checkpoint; architecture unchanged from BF16 base |

KV cache budget at gpu_memory_utilization=0.90:  72 GB − 32 GB = 40.0 GB

KV dtype BF16: 496.0 KB/token × 2048 tokens = 969 MB/session → max 41 concurrent sessions

KV dtype BF16: 496.0 KB/token × 4096 tokens = 1938 MB/session → max 20 concurrent sessions

KV dtype BF16: 496.0 KB/token × 8192 tokens = 3875 MB/session → max 10 concurrent sessions

KV dtype FP8: 248.0 KB/token × 2048 tokens = 484 MB/session → max 82 concurrent sessions

KV dtype FP8: 248.0 KB/token × 4096 tokens = 969 MB/session → max 41 concurrent sessions

KV dtype FP8: 248.0 KB/token × 8192 tokens = 1938 MB/session → max 20 concurrent sessions

Note: TRITON_ATTN constraint persists regardless of weight dtype (R8).

Note: Block FP8 weights halve VRAM (~32 GB), freeing significant headroom for KV cache

## 4.x  GLM-4.7-Flash (30B / 3.6B active MoE, MLA)

| Parameter | Value |
| --- | --- |
| Full model name | zai-org/GLM-4.7-Flash |
| Architecture type | MoE Transformer with Multi-head Latent Attention (MLA) |
| Attention detail | MLA: KV compressed to kv_lora_rank=512 per layer (vs full GQA KV space). Similar to DeepSeek-V3 MLA design. |
| KV-contributing layers | 61 |
| MLA kv_lora_rank | 512 |
| MoE | Yes |
| Mamba hybrid | No |
| MLA | Yes |
| TurboQuant compatible | Yes |
| Weight VRAM (BF16) | 60 GB |
| Confidence | ESTIMATED |
| Source | GLM-4.7-Flash architectural design inferred from zai-org model card and MLA literature |

KV cache budget at gpu_memory_utilization=0.90:  72 GB − 60 GB = 12.0 GB

KV dtype BF16: 122.0 KB/token × 2048 tokens = 238 MB/session → max 50 concurrent sessions

KV dtype BF16: 122.0 KB/token × 4096 tokens = 477 MB/session → max 25 concurrent sessions

KV dtype BF16: 122.0 KB/token × 8192 tokens = 953 MB/session → max 12 concurrent sessions

KV dtype FP8: 61.0 KB/token × 2048 tokens = 119 MB/session → max 100 concurrent sessions

KV dtype FP8: 61.0 KB/token × 4096 tokens = 238 MB/session → max 50 concurrent sessions

KV dtype FP8: 61.0 KB/token × 8192 tokens = 477 MB/session → max 25 concurrent sessions

Note: MLA compresses KV; TurboQuant would apply to the latent vector, not standard K/V heads. Compatibility unverified.

Note: BF16 weights; MLA means KV cache ~4× smaller than equivalent GQA model

## 4.x  Mistral Small 3.1 24B-Instruct

| Parameter | Value |
| --- | --- |
| Full model name | mistralai/Mistral-Small-3.1-24B-Instruct-2503 |
| Architecture type | Dense GQA Transformer |
| Attention detail | GQA 4:1 (32 Q heads / 8 KV heads), sliding window attention |
| KV-contributing layers | 40 |
| KV heads per layer | 8 |
| Head dimension | 128 |
| MoE | No |
| Mamba hybrid | No |
| MLA | No |
| TurboQuant compatible | Yes |
| Weight VRAM (BF16) | 48 GB |
| Confidence | HIGH |
| Source | Mistral Small 3.1 model card; HF config mistralai/Mistral-Small-3.1-24B-Instruct-2503 |

KV cache budget at gpu_memory_utilization=0.90:  72 GB − 48 GB = 24.0 GB

KV dtype BF16: 160.0 KB/token × 2048 tokens = 312 MB/session → max 76 concurrent sessions

KV dtype BF16: 160.0 KB/token × 4096 tokens = 625 MB/session → max 38 concurrent sessions

KV dtype BF16: 160.0 KB/token × 8192 tokens = 1250 MB/session → max 19 concurrent sessions

KV dtype FP8: 80.0 KB/token × 2048 tokens = 156 MB/session → max 153 concurrent sessions

KV dtype FP8: 80.0 KB/token × 4096 tokens = 312 MB/session → max 76 concurrent sessions

KV dtype FP8: 80.0 KB/token × 8192 tokens = 625 MB/session → max 38 concurrent sessions

Note: BF16 weights; 48 GB — substantial KV cache headroom (24 GB+)

## 4.x  Nemotron-3-Nano 30B-Instruct (Mamba hybrid)

| Parameter | Value |
| --- | --- |
| Full model name | nvidia/Nemotron-3-Nano-30B-Instruct |
| Architecture type | Mamba-2 SSM + Sparse Attention Hybrid |
| Attention detail | ~36 attention layers (with GQA 8 KV heads) alternating with Mamba-2 SSM layers. Only attention layers contribute to KV cache. |
| KV-contributing layers | 36 |
| KV heads per layer | 8 |
| Head dimension | 128 |
| MoE | Yes |
| Mamba hybrid | Yes |
| MLA | No |
| TurboQuant compatible | No |
| Weight VRAM (BF16) | 60 GB |
| Confidence | ESTIMATED |
| Source | Nemotron-3-Nano architecture from NVIDIA technical report; Mamba layer fraction estimated |

KV cache budget at gpu_memory_utilization=0.90:  72 GB − 60 GB = 12.0 GB

KV dtype BF16: 144.0 KB/token × 2048 tokens = 281 MB/session → max 42 concurrent sessions

KV dtype BF16: 144.0 KB/token × 4096 tokens = 562 MB/session → max 21 concurrent sessions

KV dtype BF16: 144.0 KB/token × 8192 tokens = 1125 MB/session → max 10 concurrent sessions

KV dtype FP8: 72.0 KB/token × 2048 tokens = 141 MB/session → max 85 concurrent sessions

KV dtype FP8: 72.0 KB/token × 4096 tokens = 281 MB/session → max 42 concurrent sessions

KV dtype FP8: 72.0 KB/token × 8192 tokens = 562 MB/session → max 21 concurrent sessions

Note: TurboQuant boundary-layer protection requires uniform attention layers. Mamba hybrid raises NotImplementedError at arg_utils.py:1650 — see CLAUDE.md R6.

Note: vLLM compat uncertain for Mamba-2 hybrid (R6). HF generate() fallback may be needed.

## 4.x  Qwen3.5-35B-A3B (DeltaNet + MoE, BF16)

| Parameter | Value |
| --- | --- |
| Full model name | Qwen/Qwen3.5-35B-A3B |
| Architecture type | DeltaNet Linear Attention + MoE Hybrid |
| Attention detail | DeltaNet linear attention layers (hardware-efficient, O(1) state per layer) interleaved with sparse standard attention layers. MoE FFN: ~3B active params. |
| KV-contributing layers | 16 |
| KV heads per layer | 4 |
| Head dimension | 128 |
| MoE | Yes |
| Mamba hybrid | No |
| MLA | No |
| TurboQuant compatible | Yes |
| Weight VRAM (BF16) | 70 GB |
| Confidence | ESTIMATED |
| Source | Qwen3.5 model card; DeltaNet architecture from Qwen team blog; KV layer count estimated |

KV cache budget at gpu_memory_utilization=0.90:  72 GB − 70 GB = 2.0 GB

KV dtype BF16: 32.0 KB/token × 2048 tokens = 62 MB/session → max 32 concurrent sessions

KV dtype BF16: 32.0 KB/token × 4096 tokens = 125 MB/session → max 16 concurrent sessions

KV dtype BF16: 32.0 KB/token × 8192 tokens = 250 MB/session → max 8 concurrent sessions

KV dtype FP8: 16.0 KB/token × 2048 tokens = 31 MB/session → max 64 concurrent sessions

KV dtype FP8: 16.0 KB/token × 4096 tokens = 62 MB/session → max 32 concurrent sessions

KV dtype FP8: 16.0 KB/token × 8192 tokens = 125 MB/session → max 16 concurrent sessions

Note: TurboQuant applies only to standard attention layers. DeltaNet state is separate. Verify boundary-layer handling.

Note: BF16 weights OOM risk: 70 GB > 72 GB usable (R1). Only viable with --max-model-len reduced or at gpu_memory_utilization < 0.90.

## 4.x  Qwen3.6-35B-A3B (DeltaNet + MoE, BF16)

| Parameter | Value |
| --- | --- |
| Full model name | Qwen/Qwen3.6-35B-A3B |
| Architecture type | DeltaNet Linear Attention + MoE Hybrid (next-gen Qwen3.5) |
| Attention detail | Same DeltaNet + MoE architecture as Qwen3.5 with expanded context (262K tokens). Multi-Token Prediction (MTP) speculative decoding. |
| KV-contributing layers | 16 |
| KV heads per layer | 4 |
| Head dimension | 128 |
| MoE | Yes |
| Mamba hybrid | No |
| MLA | No |
| TurboQuant compatible | Yes |
| Weight VRAM (BF16) | 70 GB |
| Confidence | ESTIMATED |
| Source | Qwen3.6 model card; extended-context variant of Qwen3.5-35B-A3B architecture |

KV cache budget at gpu_memory_utilization=0.90:  72 GB − 70 GB = 2.0 GB

KV dtype BF16: 32.0 KB/token × 2048 tokens = 62 MB/session → max 32 concurrent sessions

KV dtype BF16: 32.0 KB/token × 4096 tokens = 125 MB/session → max 16 concurrent sessions

KV dtype BF16: 32.0 KB/token × 8192 tokens = 250 MB/session → max 8 concurrent sessions

KV dtype FP8: 16.0 KB/token × 2048 tokens = 31 MB/session → max 64 concurrent sessions

KV dtype FP8: 16.0 KB/token × 4096 tokens = 62 MB/session → max 32 concurrent sessions

KV dtype FP8: 16.0 KB/token × 8192 tokens = 125 MB/session → max 16 concurrent sessions

Note: Same as Qwen3.5 — only standard attention layers. 262K context makes max_model_len planning critical.

Note: BF16 weights OOM risk: 70 GB > 72 GB usable (same as Qwen3.5, R1).

## 4.x  Qwen3.6-35B-A3B (DeltaNet + MoE, FP8 weights)

| Parameter | Value |
| --- | --- |
| Full model name | Qwen/Qwen3.6-35B-A3B-FP8 |
| Architecture type | DeltaNet Linear Attention + MoE Hybrid — FP8 quantized weights |
| Attention detail | Same architecture as BF16 variant; FP8 weights halve VRAM to ~35 GB |
| KV-contributing layers | 16 |
| KV heads per layer | 4 |
| Head dimension | 128 |
| MoE | Yes |
| Mamba hybrid | No |
| MLA | No |
| TurboQuant compatible | Yes |
| Weight VRAM (BF16) | 35 GB |
| Confidence | ESTIMATED |
| Source | FP8 variant of Qwen3.6-35B-A3B; architecture unchanged from BF16 |

KV cache budget at gpu_memory_utilization=0.90:  72 GB − 35 GB = 37.0 GB

KV dtype BF16: 32.0 KB/token × 2048 tokens = 62 MB/session → max 592 concurrent sessions

KV dtype BF16: 32.0 KB/token × 4096 tokens = 125 MB/session → max 296 concurrent sessions

KV dtype BF16: 32.0 KB/token × 8192 tokens = 250 MB/session → max 148 concurrent sessions

KV dtype FP8: 16.0 KB/token × 2048 tokens = 31 MB/session → max 1184 concurrent sessions

KV dtype FP8: 16.0 KB/token × 4096 tokens = 62 MB/session → max 592 concurrent sessions

KV dtype FP8: 16.0 KB/token × 8192 tokens = 125 MB/session → max 296 concurrent sessions

Note: TurboQuant on standard attention layers. FP8 weights free ample KV cache headroom.

Note: FP8 weights resolve the BF16 OOM risk; ~37 GB free for KV cache at GPU util 0.90

## 4.x  Qwen3-32B

| Parameter | Value |
| --- | --- |
| Full model name | Qwen/Qwen3-32B |
| Architecture type | Dense GQA Transformer |
| Attention detail | GQA 8:1 (64 Q heads / 8 KV heads) |
| KV-contributing layers | 64 |
| KV heads per layer | 8 |
| Head dimension | 128 |
| MoE | No |
| Mamba hybrid | No |
| MLA | No |
| TurboQuant compatible | Yes |
| Weight VRAM (BF16) | 64 GB |
| Confidence | HIGH |
| Source | Qwen3 Technical Report; HF config Qwen/Qwen3-32B |

KV cache budget at gpu_memory_utilization=0.90:  72 GB − 64 GB = 8.0 GB

KV dtype BF16: 256.0 KB/token × 2048 tokens = 500 MB/session → max 16 concurrent sessions

KV dtype BF16: 256.0 KB/token × 4096 tokens = 1000 MB/session → max 8 concurrent sessions

KV dtype BF16: 256.0 KB/token × 8192 tokens = 2000 MB/session → max 4 concurrent sessions

KV dtype FP8: 128.0 KB/token × 2048 tokens = 250 MB/session → max 32 concurrent sessions

KV dtype FP8: 128.0 KB/token × 4096 tokens = 500 MB/session → max 16 concurrent sessions

KV dtype FP8: 128.0 KB/token × 8192 tokens = 1000 MB/session → max 8 concurrent sessions

Note: BF16 weights; 64 GB — only 8 GB KV cache headroom at GPU util 0.90. FP8 KV dtype strongly recommended for meaningful concurrency.

# 5. Cross-Model Comparison

## 5.1  KV Cache Budget and Predicted Concurrency Matrix

Table below: weight VRAM from YAML vram_estimate_gb; KV budget = 72 GB − weight_VRAM; max concurrent sessions at ctx=4096 = floor(kv_budget / kv_per_session).  Values are conservative lower bounds.

| Model | Weight VRAM (GB) | KV budget (GB) | BF16 KV/token (KB) | Max sessions BF16 | Max sessions FP8 KV | Max sessions TQ-3bit |
| --- | --- | --- | --- | --- | --- | --- |
| Gemma 3 27B-IT | 54 | 18.0 | 992.0 | 4 | 9 | 24 |
| Gemma 4 26B-A4B-IT (MoE, BF16) | 52 | 20.0 | 368.0 | 13 | 27 | N/A |
| Gemma 4 26B-A4B-IT (MoE, FP8 weights) | 26 | 46.0 | 368.0 | 32 | 64 | N/A |
| Gemma 4 31B-IT (Dense, BF16) | 62 | 10.0 | 496.0 | 5 | 10 | N/A |
| Gemma 4 31B-IT (Dense, FP8 block weights) | 32 | 40.0 | 496.0 | 20 | 41 | N/A |
| GLM-4.7-Flash (30B / 3.6B active MoE, MLA) | 60 | 12.0 | 122.0 | 25 | 50 | 134 |
| Mistral Small 3.1 24B-Instruct | 48 | 24.0 | 160.0 | 38 | 76 | 204 |
| Nemotron-3-Nano 30B-Instruct (Mamba hybrid) | 60 | 12.0 | 144.0 | 21 | 42 | N/A |
| Qwen3.5-35B-A3B (DeltaNet + MoE, BF16) | 70 | 2.0 | 32.0 | 16 | 32 | 85 |
| Qwen3.6-35B-A3B (DeltaNet + MoE, BF16) | 70 | 2.0 | 32.0 | 16 | 32 | 85 |
| Qwen3.6-35B-A3B (DeltaNet + MoE, FP8 weights) | 35 | 37.0 | 32.0 | 296 | 592 | 1578 |
| Qwen3-32B | 64 | 8.0 | 256.0 | 8 | 16 | 42 |

## 5.2  TurboQuant Compatibility Matrix

| Model | TQ Compatible? | Reason |
| --- | --- | --- |
| Gemma 3 27B-IT | ✓ Yes | Compatible |
| Gemma 4 26B-A4B-IT (MoE, BF16) | ✗ No | vLLM forces TRITON_ATTN (heterogeneous head_dim > 256 mixed with 128 local layers). TRITON_ATTN does not support turboquant_* — see CLAUDE.md R8. |
| Gemma 4 26B-A4B-IT (MoE, FP8 weights) | ✗ No | Same TRITON_ATTN constraint as BF16 variant (R8); FP8 KV cache also blocked. |
| Gemma 4 31B-IT (Dense, BF16) | ✗ No | vLLM forces TRITON_ATTN (heterogeneous head_dim) — same constraint as A4B variants (R8). |
| Gemma 4 31B-IT (Dense, FP8 block weights) | ✗ No | TRITON_ATTN constraint persists regardless of weight dtype (R8). |
| GLM-4.7-Flash (30B / 3.6B active MoE, MLA) | ✓ Yes | MLA compresses KV; TurboQuant would apply to the latent vector, not standard K/V heads. Compatibility unverified. |
| Mistral Small 3.1 24B-Instruct | ✓ Yes | Compatible |
| Nemotron-3-Nano 30B-Instruct (Mamba hybrid) | ✗ No | TurboQuant boundary-layer protection requires uniform attention layers. Mamba hybrid raises NotImplementedError at arg_utils.py:1650 — see CLAUDE.md R6. |
| Qwen3.5-35B-A3B (DeltaNet + MoE, BF16) | ✓ Yes | TurboQuant applies only to standard attention layers. DeltaNet state is separate. Verify boundary-layer handling. |
| Qwen3.6-35B-A3B (DeltaNet + MoE, BF16) | ✓ Yes | Same as Qwen3.5 — only standard attention layers. 262K context makes max_model_len planning critical. |
| Qwen3.6-35B-A3B (DeltaNet + MoE, FP8 weights) | ✓ Yes | TurboQuant on standard attention layers. FP8 weights free ample KV cache headroom. |
| Qwen3-32B | ✓ Yes | Compatible |

# 6. Empirical Benchmark Methodology

Section 4's predictions are based on analytical memory formulas and published architecture parameters.  The empirical benchmark measures actual TTFT, TPOT, ITL, throughput, and peak VRAM under real load, accounting for vLLM's scheduling overhead, CUDA kernel launch latency, and prefill/decode pipeline effects.

## 6.1  Script

scripts/run_concurrency_benchmark.sh — single model, single dtype.

./scripts/run_concurrency_benchmark.sh configs/models/cat_a/qwen3_32b.yaml \
  --kv-cache-dtype fp8 \
  --context-lengths 2048,4096,8192 \
  --concurrency-levels 1,2,4,8,16,32,64,128,256,512,1024 \
  --requests-per-level 64 \
  --output-tokens 128

## 6.2  Metrics Captured Per Level

| Metric | Definition |
| --- | --- |
| TTFT p50/p95/p99 (ms) | Time from request submit to first output token |
| TPOT p50/p95/p99 (ms) | (E2E − TTFT) / (output_tokens − 1): per-token decode time |
| ITL p50/p95/p99 (ms) | Inter-token latency for each successive output token |
| E2E latency p50/p95/p99 (ms) | Total request round-trip time |
| Throughput (output tok/s) | Aggregate output tokens / wall-clock time for the level |
| Goodput (req/s) | Successful requests / wall-clock time |
| Success rate | Fraction of requests completing without error |
| Peak VRAM (GB) | Maximum GPU memory observed during the level (nvidia-smi polling) |

## 6.3  Prompt Shape

Input: ~512 tokens (business-workflow description, looped to fill the target size).  Output: max_tokens=128.  Temperature=0.0 (greedy; maximises reproducibility).  Warmup: 8 requests discarded per level to allow KV cache warm-up.

# 7. Risks and Known Deviations

| ID | Risk | Description | Mitigation |
| --- | --- | --- | --- |
| R1 | Qwen3.5/3.6-35B-A3B BF16 OOM | 70 GB weights > 72 GB usable; only viable at reduced max_model_len or lower gpu_memory_utilization | Use FP8 variant (Qwen3.6-35B-A3B-FP8) or reduce max_model_len to 2048 |
| R6 | Nemotron-3-Nano vLLM incompatibility | Mamba-2 hybrid may not be supported by vLLM's attention backend for all dtypes | HF generate() fallback; skip TurboQuant dtypes; benchmark with --kv-cache-dtype auto only |
| R8 | Gemma4 + TurboQuant incompatible | vLLM forces TRITON_ATTN for heterogeneous head_dim; TRITON_ATTN rejects turboquant_* dtype | Use FP8 / KIVI / KVQuant for all Gemma4 cells in the benchmark matrix |
| R-MLA | GLM-4.7-Flash MLA KV format | MLA's compressed KV latent may not be compatible with all KV dtype options in vLLM | Benchmark with auto and fp8 first; verify MLA passthrough for TurboQuant |
| R-OOM2 | Qwen3-32B BF16 KV headroom | Only 8 GB KV budget at BF16; max_model_len must be small to allow >1 concurrent session | Always benchmark Qwen3-32B with --kv-cache-dtype fp8 or turboquant_3bit_nc |

# Appendix C: Empirical Results (TBD)

This appendix is a placeholder.  Populate rows by running the benchmark script for each (model, dtype) pair and aggregating results/concurrency/*.json.

Command to run all Cat A models at fp8 dtype (example):

for cfg in configs/models/cat_a/*.yaml; do
  ./scripts/run_concurrency_benchmark.sh "$cfg" --kv-cache-dtype fp8
done

| Model | KV dtype | ctx=2048 max conc. | ctx=4096 max conc. | ctx=8192 max conc. | TTFT p95 @baseline (ms) | Peak VRAM (GB) |
| --- | --- | --- | --- | --- | --- | --- |
| Gemma 3 27B-IT | fp8 | TBD | TBD | TBD | TBD | TBD |
| Gemma 4 26B-A4B-IT (MoE, BF16) | fp8 | TBD | TBD | TBD | TBD | TBD |
| Gemma 4 26B-A4B-IT (MoE, FP8 weights) | fp8 | TBD | TBD | TBD | TBD | TBD |
| Gemma 4 31B-IT (Dense, BF16) | fp8 | TBD | TBD | TBD | TBD | TBD |
| Gemma 4 31B-IT (Dense, FP8 block weights) | fp8 | TBD | TBD | TBD | TBD | TBD |
| GLM-4.7-Flash (30B / 3.6B active MoE, MLA) | fp8 | TBD | TBD | TBD | TBD | TBD |
| Mistral Small 3.1 24B-Instruct | fp8 | TBD | TBD | TBD | TBD | TBD |
| Nemotron-3-Nano 30B-Instruct (Mamba hybrid) | fp8 | TBD | TBD | TBD | TBD | TBD |
| Qwen3-32B | fp8 | TBD | TBD | TBD | TBD | TBD |
| Qwen3.5-35B-A3B (DeltaNet + MoE, BF16) | fp8 | TBD | TBD | TBD | TBD | TBD |
| Qwen3.6-35B-A3B (DeltaNet + MoE, BF16) | fp8 | TBD | TBD | TBD | TBD | TBD |
| Qwen3.6-35B-A3B (DeltaNet + MoE, FP8 weights) | fp8 | TBD | TBD | TBD | TBD | TBD |